AMX Virtual devices Module Interface Specification

for the

Lumens PTZ Camera

Revision History

| Date | Initials | Revision | Comments |
| --- | --- | --- | --- |
| 04/28/17 | SP | 1.0.0 | Initial release. |

Introduction

This is a reference manual to describe the interface provided between an AMX NetLinx system and a Lumens PTZ Camera. The Lumens PTZ Camera supports an RS-232 serial protocol. The required communication settings are a baud rate of 9600/38400, 8 data bits, 1 stop bit, no parity, and handshaking off. The module defaults to 9600 baud rate.

Cable ( AMX RS-232 To Lumens Camera 1 RS-232 In )

AMXINI(DB-9 Female)				   Camera 1 RS-232 In (Mini Din 8 Male)

2 TXD          						   5 RXD

3 RXD           					   3 TXD

5 GND           					   4 GND

Cable (Lumens Camera 1 RS-232 Out To Lumens Camera 2 RS-232 In )

Camera 1 RS-232 Out (Mini Din 8 Male)	Camera 2 RS-232 In (Mini Din 8 Male)

1 DTX								    2 DSR

2 DSR								    1 DTX

3 TXD								    5 RXD

4 GND								    4 GND

5 RXD								    3 TXD

6 GND								    6 GND

This module support AMX NX Series Controller.

This module was written using NetLinx Studio version 4.3, build 4.3.1519.

Overview

The Lumens_PTZ_Camera_Module translates between the standard interface described below and the camera serial protocol. It parses the buffer for responses from the camera, sends strings to control the camera, and receives commands from the UI module or telnet sessions.

A User Interface (UI) module is also provided. This module uses the standard interface described below and parses the command responses for feedback.

The following diagram gives a graphical view of the interface between the interface code and the Virtual device.

Lumens PTZ Camera

NetLinx

UI Module

Virtual

Device

A sample UI module and a touch panel file are provided in the module package.  These are not intended to cover every possible application, but can be expanded as needed by a dealer to meet the requirements of a particular installation.

Implementation

To interface to the AMX Lumens PTZ Camera module, the programmer must perform the following steps:

- Define the device ID for the camera that will be controlled.

- Define the virtual device ID that the Lumens PTZ Camera COMM module will use to communicate with the main program and User Interface. Virtual devices use device numbers 32768-36864.

- If a touch panel interface is desired, a touch panel file Lumens_PTZ_Camera_UI.TP4 and module (Lumens_Module.apw) have been created for testing.

- The Virtual Lumens PTZ Camera module must be included in the program with a DEFINE_MODULE command. This command starts execution of the module and passes in the following key information: the device ID of the camera to be controlled, and the virtual device ID for communicating to the main program.

An example of how to do this is shown below

DEFINE_DEVICE

dv_Lumens	=	5001:1:0

// This device should be used as the physical device by the COMM module

vdv_Lumens	=	33001:1:0

// The COMM module should use this as its Virtual Device

dvTP_Lumens	=	10001:1:0

// This port should match the assigned touch panel device port

DEFINE_START

define_module 'Lumens_PTZ_Camera_Module' Lumens_Module(vdv_Lumens,dv_Lumens,dvTP_Lumens)

Port Mapping

| Virtual Device | Channels | Levels | Control | Feedback |
| --- | --- | --- | --- | --- |
| 33001:1:0 - camera 1 - 7 | All Channels | Applicable Levels | Applicable to all Control Commands | Applicable to all Feedback Commands |

Table 1 - Port Mapping

Channels

The UI module controls the Lumens PTZ Camera via channel events (NetLinx commands pulse, on, and off) sent to the Lumens_PTZ_Camera_Module. The channels supported by the Lumens_PTZ_Camera_Module are listed below. These channels are associated with the virtual device(s) and are independent of the channels associated with the touch panel device.

| Channel | Description |
| --- | --- |
| 1 | PULSE: Set Power on |
| 2 | PULSE: Set Power off |
| 11 | PULSE: Set Select Camera 1 |
| 12 | PULSE: Set Select Camera 2 |
| 13 | PULSE: Set Select Camera 3 |
| 14 | PULSE: Set Select Camera 4 |
| 15 | PULSE: Set Select Camera 5 |
| 16 | PULSE: Set Select Camera 6 |
| 17 | PULSE: Set Select Camera 7 |
| 100 | PULSE: Set Main Page Select Normal Popup Pages |
| 101 | PULSE: Set Back Light |
| 102 | PULSE: Set Pan/Tilt Reset |
| 103 | PULSE: Set Freeze |
| 111 | PULSE: Set Pan Speed Auto |
| 112 | PULSE: Set Tilt Speed Auto |
| 200 | PULSE: Set Main Page Select Presets Pages |
| 201 | PULSE: Set Home |
| 202 | ON: Ramp Tilt Up - provides feedback also OFF: Stop Tilt Up Ramping |
| 203 | ON: Ramp Tilt Down - provides feedback also OFF: Stop Tilt Down Ramping |
| 204 | ON: Ramp Pan Left - provides feedback also OFF: Stop Pan Left Ramping |
| 205 | ON: Ramp Pan Right - provides feedback also OFF: Stop Pan Right Ramping |
| 206 | ON: Ramp Zoom In - provides feedback also OFF: Stop Zoom In Ramping |
| 207 | ON: Ramp Zoom Out - provides feedback also OFF: Stop Zoom Out Ramping |
| 208 | PULSE: Set Save |
| 209 | PULSE: Set Reset |
| 211 | PULSE: Set Preset 01 |
| 212 | PULSE: Set Preset 02 |
| 213 | PULSE: Set Preset 03 |
| 214 | PULSE: Set Preset 04 |
| 215 | PULSE: Set Preset 05 |
| 216 | PULSE: Set Preset 06 |
| 217 | PULSE: Set Preset 07 |
| 218 | PULSE: Set Preset 08 |
| 219 | PULSE: Set Preset 09 |
| 220 | PULSE: Set Preset 10 |
| 221 | PULSE: Set Preset 11 |
| 222 | PULSE: Set Preset 12 |
| 223 | PULSE: Set Preset 13 |
| 224 | PULSE: Set Preset 14 |
| 225 | PULSE: Set Preset 15 |
| 226 | PULSE: Set Preset 16 |
| 227 | PULSE: Set Preset 17 |
| 228 | PULSE: Set Preset 18 |
| 229 | PULSE: Set Preset 19 |
| 230 | PULSE: Set Preset 20 |
| 231 | PULSE: Set Preset 21 |
| 232 | PULSE: Set Preset 22 |
| 233 | PULSE: Set Preset 23 |
| 234 | PULSE: Set Preset 24 |
| 235 | PULSE: Set Preset 25 |
| 236 | PULSE: Set Preset 26 |
| 237 | PULSE: Set Preset 27 |
| 238 | PULSE: Set Preset 28 |
| 300 | PULSE: Set Main Page Select Advanced Pages |
| 301 | PULSE: Set Focus - Auto |
| 302 | PULSE: Set Focus - Manual |
| 303 | PULSE: Set Focus - One Push |
| 304 | ON: Ramp Focus - Far - - provides feedback also OFF: Stop Focus - Far - Ramping |
| 305 | ON: Ramp Focus - Near + - provides feedback also OFF: Stop Focus - Near + Ramping |
| 306 | PULSE: Set Mirror |
| 307 | PULSE: Set Flip |
| 308 | PULSE: Set Pan Tilt Speed Mode - Normal |
| 309 | PULSE: Set Pan Tilt Speed Mode - Smooth |
| 310 | PULSE: Set Preset Speed Mode - Fast |
| 311 | PULSE: Set Preset Speed Mode - Middle |
| 312 | PULSE: Set Preset Speed Mode - Slow |
| 313 | PULSE: Set Motionless Preset - Off |
| 314 | PULSE: Set Motionless Preset - On |
| 315 | PULSE: Set Tally Lamp - Off |
| 316 | PULSE: Set Tally Lamp - On |
| 317 | PULSE: Set Tally Mode - Off |
| 318 | PULSE: Set Tally Mode - On (Low) |
| 319 | PULSE: Set Tally Mode - On (High) |

Table 2 - Virtual Device Channel Events

Levels

The UI module controls the Lumens PTZ Camera via level events (NetLinx command send_level) sent to the Lumens_PTZ_Camera_Module. The levels supported by the Lumens_PTZ_Camera_Module are listed below. These levels are associated with the virtual device(s) and are independent of the levels associated with the touch panel device.

| Level | Description |
| --- | --- |
| 1 | Camera Select (range 1…7) |
| 2 | Pan Speed (range 1…24) |
| 3 | Tilt Speed (range 1…24) |
| 4 | Zoom Speed (range 1…7) |

Table 3 - Virtual Device Level Events