Partner: Lumens Digital Optics Inc. Model: LC200

Device Type: CaptureVision Station

| GENERAL INFORMATION |  |
| --- | --- |
| SIMPLWINDOWS NAME: | Lumens LC200_v2 |
| CATEGORY: | CaptureVision Station |
| VERSION: | 2.0 |
| SUMMARY: | This module will control the Lumens LC200. It uses 2-way RS232 communications, control using Crestron Touch Screen and Crestron App. |
| GENERAL NOTES: |  |
| CRESTRON HARDWARE REQUIRED: | - Crestron 3-Series Controller - Com port x1 - LAN port x1 |
| SETUP OF CRESTRON HARDWARE: | RS232/RS485 Baud:9600 Parity: None Data Bits: 8 Stop Bits: 1 TCP IP address : Media Station’s WAN IP address Port : 5080 |
| VENDOR FIRMWARE: | none |
| VENDOR SETUP: |  |

Partner: Lumens Digital Optics Inc. Model: LC200

Device Type: CaptureVision Station

| CABLE DIAGRAM: | - RS-232 / RS-485 Connect the RS-232 cable to the RS-232 upper port of the media station. Connect the RS-485 cable to the RS-485 lower port of the media station. The pin definition of the RS-232 / RS-485 port： - GND：Ground RX：Receive Data TX：Transmit Data T/R+ : D+ T/R- : D- - TCP Connect the CAT-5 (or greater) cable to WAN (or LAN) RJ-45 port of Media Station. |
| --- | --- |

| CONTROL: |  |  |
| --- | --- | --- |
| Initialize | D | Get the status from LC200 |
| Power_off | D | Power off LC200 |
| Record_Start | D | Start Record |
| Record_Stop |  | Stop Record |
| Record_get |  | Get the Record status from LC200 |

Partner: Lumens Digital Optics Inc. Model: LC200

Device Type: CaptureVision Station

| Snapshot | D | Snapshot |
| --- | --- | --- |
| Layout | D | Layout ID1~12 select |
| Layout_get | D | Get Layout ID sata |
| Background | D | Background off/1~9 select |
| Background_get | D | Get Background sata |
| Overlay | D | Overlay off/1~9 select |
| Overlay_get | D | Get Overlay sata |
| Scenes | D | Scenes 1~9 select |
| Audio_Volume | A | Set Audio Volume in1~4 and out Level |
| Audio_Volume_get | D | Get Audio_Volume in1~4 and out Level |
| Audio_mute | D | Mute in1~4 and out |
| Audio_mute_get | D | Get Audio Mute data from in1~4 and out |
| Audio_Type | D | Audio Type in1~4 and out |
| Audio_Type_get | D | Get Audio Type data from in1~4and out |
| Stream | D | Set Stream1~3 |
| Stream_get | D | Get Stream1~3 data |
| Camera select | D | Camera 1~4 select |
| Camera Up/Down/Left/Right | D | Camera Up/Down/Left/Right |
| Camera_Move_speed_1~18 | A | Set Camera Up/Down/Left/Right Move Speed 1~18 |
| Camera Zoom In/Out | D | Camera Zoom In/Out |
| Camera_zoom_speed_1~7 | A | Set Camera Zoom In/Out Speed 1~7 |

Partner: Lumens Digital Optics Inc. Model: LC200

Device Type: CaptureVision Station

| Camera_Preset_call | D | Call Camera Preset1~9 |
| --- | --- | --- |
| Camera_Preset_save | D | Save Camera Preset1~9 |
| Switch_CH1~4_Trig | D | Video source Channel1~4 select |
| Switch_CH1~4_Parameter | A | Set Video source Channel1~4 Parameter |
| Switch_Channel_Source_get | D | Get Video source Channel1~4 select and Parameter |
| Macro | D | Marco 1~3 select |

| FEEDBACK: |  |  |
| --- | --- | --- |
| Initialize_Busy_fb | D | The feedback of initializing |
| Record_State_Text | S | Record State Information |
| Layout_ID_fb | D | The feedback of Layout ID |
| Background _fb | D | The feedback of Background |
| Overlay _fb | D | The feedback of Overlay ID |
| Scene _fb | D | The feedback of Scene ID |
| Audio_Volume_Level_dB | A | The feedback of Volume Level dB ( 0~125 ) |
| Audio_Volume_Level_gain | A | The feedback of Volume Level gain ( 0~65535 ) |
| Audio_mute/unmute_fb | D | The feedback of Audio Mute data |
| Audio_Type _fb | D | The feedback of Audio Type data |
| Stream _Fb | D | The feedback of Stream data |
| Camera_select_fb | D | The feedback of camera select 1~4 |
| Camera_Preset_fb | D | The feedback of Camera Preset 1~9 |
| Switch_CH_Parameter_fb | A | The feedback of Video_Source_ID Channel Parameter |
| Macro_fb | D | The feedback of Marco 1~3 |

Partner: Lumens Digital Optics Inc. Model: LC200

Device Type: CaptureVision Station

| TESTING: |  |
| --- | --- |
| OPS USED FOR TESTING: | CP3: v1.012.0017 |
| SIMPL WINDOWS USED FOR TESTING: | 4.11.05 |
| DEVICE DB USED FOR TESTING: | 114.00.001.00 |
| CRES DB USED FOR TESTING: | 88.00.002.00 |
| SYMBOL LIBRARY USED FOR TESTING: | 1101 |
| SAMPLE PROGRAM: | 3-Series: CP3-LC200_v3.smw |
| REVISION HISTORY: | V2.0 |